# Digital Accessibility: How to make your Kiosk or Bank ATM Accessible

# How to make your Kiosk / Bank ATM Accessible

- Physical location of the Kiosk / ATM must be accessible by everyone including elderly and people with disabilities

- Ensure people with disabilities can physically get to the Kiosk machine from anywhere in the building

- Reception staff must be available and be prepared to support and explain the location of the Kiosk machine

- Clear signage with good lighting, high color contrast, large Arial or Verdana fonts should be used for people with low vision

- Place kiosk in a location free of loud noise and glare on the kiosk screen from various light sources

- Kiosk / ATM interface hardware and software must be bilingual, Arabic and English Languages. The screen reader must also be available in both Languages

- Person must be able to access Kiosk / ATM control panel from a 
     seated position; Wheelchair

- The Kiosk operating panel must be within 48-inch reach for persons seated in a wheelchair, forward and side reach

- Ensure controls located around display screen are also within reach; 
48-inch reach

- Provide overhang style control panel so wheelchairs users may be able to wheel under it with knees and thigh for optimal reach of all controls

- Kiosk / ATM must be equipped with tactile and Audio Jack

- Audio jack must be placed in a consistent location on the control panel with tactile earphone plug and volume button control

- Kiosk control buttons and functions must have Tactile and Braille

- The Kiosk / ATM Application must have options for built in assistive technologies for blind and low vision users

- Screen reader must function when earphone jack is plugged in

- Kiosk Application must be fully accessible by screen reading technology

- Screen reader users should be able to adjust the volume of the speech

- Optionally: Screen Magnifier application can be built into the kiosk application interface and the button to open it should be easily located

- Use mobile SMS notification of bank transactions along with paper receipt 
     from Kiosk machines

- Banks notify all transactions to customers through mobile SMS

- Mobile SMS is fully accessible to screen reader users

- Accessible text in Kiosk / ATM Application

- Use accessible Text, Sans-serif fonts; examples; Verdana, Arial, Tahoma

- Minimum text size should be between 18 and 24 points for Arabic and English Languages

- Avoid using stylized text such as drop shadow, italics, strike through

- When emphasizing words or phrases, use Bold text with italic

- Avoid using Flashing, Blinking, Animated or Moving text

- Foreground (Text) Color should be used with High Color contrast and not 
     used alone to convey a meaning, prompt an action or response

- Avoid using color alone to convey a meaning, Example; Avoid using Red color alone to convey a error message

- Minimum color contrast between text and background should be 4.5:1

- Minimum color contrast between graphic elements and background 
should be 3:1

- When Audio and Videos are used, provide text equivalents

- When using audio; provide text transcript

- When using videos; ensure there are subtitles and or audio description / narration when there is no spoken dialogue

- Kiosks / ATMs used for financial Transactions

- Ensure universally accessible numeric keypads are used with the raised dot tactile on the 5 key and tactile  “O” for Okay and “X” for cancel

- Use accessible credit and bank card slots seen on ADA compliant ATM machines

References:

Follow and use the ADA and WCAG 2.2 guidelines to ensure your Kiosk / ATM machine is accessible to people with disabilities.